OpenVetSim v2.6.2

Windows: Build, Install and Test

Follow the parts in order. Each step says what you should see, so you can confirm it worked before moving on.

Two things to know before you start 1. The simulation manager must be on WIRED Ethernet (or 5 GHz Wi-Fi). On 2.4 GHz you will get multi-second dropouts no matter what the software does. 2. You do NOT need to update the BeagleBone controller first. v2.6.2 is fully backward compatible with existing controller firmware — it connects and runs normally. Updating the controller as well removes the remaining timing jitter, but it is a separate job and can wait. See CONTROLLER_UPDATE_CHECKLIST.md.

Allow about an hour, most of it in Part 6 (testing). The build itself is quick.

| Part | What you do | Time |
| --- | --- | --- |
| Part 1 | Save your rollback installer | 5 min |
| Part 2 | Get the code and verify it | 5 min |
| Part 3 | Compile the C++ binary | 10 min |
| Part 4 | Build the installer | 10 min |
| Part 5 | Install it | 5 min |
| Part 6 | Test — the important part | 20 min |
| Part 7 | If something is wrong: roll back | as needed |
| Part 8 | If it works: publish the release | 5 min |

Part 1 · Save your rollback installer

Do this first. If anything goes wrong on Tuesday, this is what gets you teaching again in five minutes rather than rebuilding under pressure.

1.1  Copy the existing 2.6.1 installer somewhere safe

In PowerShell, from the repository folder:

$dest = "$env:USERPROFILE\Desktop\OpenVetSim-rollback" mkdir $dest copy "OpenVetSim-App\dist\OpenVetSim Setup 2.6.1.exe" $dest

If that file is not there It is also published on GitHub. Go to the repository, then Releases, then v2.6.1, and download OpenVetSim.Setup.2.6.1.exe. Save it to the same Desktop folder.

1.2  Confirm the controller is ready

☐  The BeagleBone controller is powered on

☐  The PC is connected by wired Ethernet, not Wi-Fi

☐  You can see the manikin and hear its speaker from where you are sitting

You need to hear the heart sound in Part 6, so sit somewhere you can.

Part 2 · Get the code and verify it

The changes are on a branch called beat-timing-fixes, not on main. If you build from main you will get none of this.

2.1  Pull the branch

cd $env:USERPROFILE\Documents\"Claude OVS" git fetch origin git checkout beat-timing-fixes git pull

2.2  Verify you actually have the right code

This check takes ten seconds and prevents the single most wasteful mistake — building the wrong code and then testing it for an hour.

git log --oneline -1 findstr version OpenVetSim-App\package.json findstr /C:"sim_monotonic_msec" OpenVetSim\platform.h ¦ find /c "sim_monotonic_msec" findstr /C:"drainListeners" OpenVetSim\pulse.cpp ¦ find /c "drainListeners"

What you should see:

74b113d Bump version to 2.6.2

"version": "2.6.2",

3   (or more)

4   (or more)

Stop if any of these is wrong A count of 0, or a version of 2.6.1, means you are not on the right code. Run git branch --show-current and confirm it says beat-timing-fixes. Do not continue until all four checks pass.

Part 3 · Compile the C++ binary

3.1  Build it

cd OpenVetSim\build cmake .. -DCMAKE_BUILD_TYPE=Release cmake --build . --config Release

Look for "Build succeeded" with 0 errors. Warnings are normal and can be ignored.

3.2  Make sure the binary is where Electron expects it

This one catches people. Visual Studio often writes the binary into a Release subfolder, but the app looks for it one level up. Check:

dir bin\WinVetSim.exe

If that says "File Not Found", copy it up from the Release folder:

copy bin\Release\WinVetSim.exe bin\

Confirm it now exists and is dated today. If the app cannot find it later, it pops up a "Binary not found" message with the full path it looked in.

3.3  Confirm PHP is present

The installer bundles PHP from this folder. If you have built on this PC before it will already be there.

dir bin\PHP8.0\php.exe

If PHP is missing Run .\scripts\download-php.ps1 from the repository root. Note the download CDN is blocked on university networks, so you may need a home connection or a phone hotspot. End users are unaffected — PHP is bundled inside the installer.

Part 4 · Build the installer

4.1  Install the Node dependencies

v2.6.2 adds a new dependency (ssh2, used by the controller survey), so this step is required this time even if you have skipped it before.

cd ..\..\OpenVetSim-App npm install

4.2  Build it

npm run dist:win

4.3  Confirm the version number

dir dist\*.exe

You should see OpenVetSim Setup 2.6.2.exe. If it says 2.6.1, the version bump did not come through — go back to Part 2.

Your rollback copy is safe The new installer is named 2.6.2, so it cannot overwrite the 2.6.1 installer you saved in Part 1.

4.4  Signing — optional for this test

An unsigned installer works fine; Windows will show a SmartScreen warning that you can click through. You only need to sign for distribution to others. If you do want to sign, insert the YubiKey first and run the signtool command from CLAUDE.md — it will prompt for the PIN once.

Part 5 · Install it

☐  Close OpenVetSim if it is running

☐  Run dist\OpenVetSim Setup 2.6.2.exe

☐  Click through the SmartScreen warning if it appears (More info, then Run anyway)

☐  Let it install over the existing version

Then start OpenVetSim and confirm the controller connects — the status dot in the instructor interface should turn green.

Part 6 · Test — the important part

This is where you find out whether to teach on 2.6.2 or fall back to 2.6.1. Budget twenty minutes and do not skip it.

Why this matters more than usual v2.6.2 rewrote the part of the code that accepts controller connections, so that it can listen on the old port 50200 as well as 40844. That code has been tested on a Mac but has never run on Windows. It is the one part of this release that could behave differently here.

6.1  Watch the log while it runs

The messages you need come from the simulation engine. The simplest way to see them is to run the app from a terminal instead of the Start menu:

cd $env:USERPROFILE\Documents\"Claude OVS"\OpenVetSim-App npm start

Engine messages appear prefixed with [sim]. Leave this window visible.

6.2  Let it run for at least 10 minutes

With the controller connected and a heart rate set. Ten minutes matters: the disconnect bug this release fixes used to appear within a minute or two, so a short test proves nothing.

Watch for these three things:

| Message | What it means |
| --- | --- |
| Close listener | Should NEVER appear. This was the disconnect bug. If you see it, the fix is not working. |
| ReOpened | Should NOT appear after the first connection. If it repeats, the controller is connecting twice and churning. |
| Controller connected | Should appear ONCE, saying port 40844. Seeing it repeatedly means reconnect churn. |

6.3  Listen at 150 beats per minute

Set the heart rate to 150 and listen to the manikin for a minute or two.

Use 150, not 120 The original timing bug was invisible at 120 and 240 BPM and worst at 150, because of how the old clock resolution lined up with those intervals. Testing at 120 tells you nothing.

6.4  Pass or fail

It passes if all of these are true:

☐  No "Close listener" messages in 10+ minutes

☐  No repeated "ReOpened" or "Controller connected" messages

☐  Heart sound is steady at 150 BPM, with no stumble or skipped beat

☐  The controller status dot stayed green throughout

If all four pass, teach on 2.6.2. If any fail, go to Part 7 — and send me the [sim] output, because that is exactly the information needed to fix it.

Part 7 · If something is wrong: roll back

Rolling back takes about five minutes. There is no shame in teaching on 2.6.1 — it already has all the beat-timing fixes. What it lacks is the socket drain, so you may see an occasional dropped beat where the controller reconnects.

7.1  Uninstall 2.6.2

Windows Settings, then Apps, find OpenVetSim, and uninstall.

7.2  Reinstall 2.6.1

Run the installer you saved in Part 1:

$env:USERPROFILE\Desktop\OpenVetSim-rollback\"OpenVetSim Setup 2.6.1.exe"

7.3  Confirm

☐  OpenVetSim starts and the controller connects (green dot)

☐  Heart sound works

Your scenarios and settings are safe Scenarios, logs and video live in C:\Users\Public\OpenVetSim and are not touched by installing or uninstalling. The manikin’s RFID tag table lives on the BeagleBone and is not affected either.

Part 8 · If it works: publish the release

Only after Part 6 passes. This puts the installer on GitHub so it becomes the version other people get, and so you have a downloadable copy for next time.

cd $env:USERPROFILE\Documents\"Claude OVS" gh release create v2.6.2 ` "OpenVetSim-App\dist\OpenVetSim Setup 2.6.2.exe" ` --title "v2.6.2" ` --notes "Heart beat timing fixes. Fully compatible with existing controller firmware - no controller update needed to install or run. Updating the controller as well removes the remaining timing jitter; see CONTROLLER_UPDATE_CHECKLIST.md. Requires wired Ethernet or 5GHz Wi-Fi for the simulation manager."

The v2.6.2 tag already exists on GitHub, so this attaches the installer to it and marks it as the latest release.

Quick reference

| Item | Value |
| --- | --- |
| Branch to build | beat-timing-fixes (not main) |
| Expected commit | 74b113d Bump version to 2.6.2 |
| Installer name | OpenVetSim Setup 2.6.2.exe |
| Rollback installer | OpenVetSim Setup 2.6.1.exe (Desktop\OpenVetSim-rollback) |
| Test heart rate | 150 BPM |
| Minimum test time | 10 minutes |
| Must never appear | Close listener |
| Must appear once only | Controller connected on port 40844 |

Recommended follow-up, but not urgent: updating the BeagleBone controller firmware removes the remaining timing jitter. v2.6.2 works fine without it — the heart sound is much improved but may still stumble occasionally at rates such as 150 BPM. See CONTROLLER_UPDATE_CHECKLIST.md and TEST_UPDATE_189.md when you are ready.